Designing and Simulating a simple RF waveguide in COMSOL

Step by Step Instructions

Sumukh Vaidya

May 24, 2025

Note: These instruction are for COMSOL Multiphysics 6.1.

- Open COMSOL. Click Model Wizard under New. Then Click 3D in space dimensions.

- Double Click Electromagnetic Waves, Frequency Domain (emw) under Select Physics.

- Click study, then click Frequency Domain.

- This opens up an empty project. Now we define geometry. The rough parameters for the stripline to maintain a ~50Ω characteristic impedance are as follows: (Use the Coplanar Waveguide Calculator on https://www.microwaves101.com/calculators/864-coplanar-waveguide-calculator

- Write the following parameters into the parameters table. These will specify geometry.

- Right click Geometry-> Work Plane. The click build selected with default values.

- Right click Plane Geometry -> Rectangle.

- Make the rectangle to denote the stripline. Make sure to center the base under Position.

- Repeat this process to implement the right and left side gold patches. Need to specify center offsets properly. Also define a rectangle encompassing the full substrate. This will be needed to define the solid block of substrate later.

- The 2D geometry should now look like this:

- Now add the sma connector. Right click Geometry-> Parts -> Part Libraries

- Find the sma connector under RF Module -> Connectors. Double click to add. Add total of 2 connectors to the geometry (for input and output).
.

- The part is loaded from library. Only change orientation and position. Do not change any other properties. Click Build Selected.

- Repeat the same for the other connector.

- Now define 2 boxes under Geometry -> Block. One will be the air volume encompassing the entire object, the other sapphire substrate block

- Click Form Union -> Build Selected. This finalizes the geometry.

- Now add materials: Air, Gold, Sapphire. Right Click Materials -> Add Material from Library. Search Air, use Built-in -> Air.

- We will add Sapphire as a blank material because it is not available in the standard catalog. Right Click Materials -> Blank material. Name it Sapphire. Select Electromagnetic Models -> dielectric Losses. Input this screenshot values.

- Right Click Electromagnetic Waves, Frequency Domain (emw) -> Perfect Electric Conductor

- This creates a Perfect Electric Conductor 2. Select all the metal faces and the surfaces of the SMA Connectors. This is what we use instead of defining Gold as a material (Au is a good conductor anyway).

- Right Click Electromagnetic Waves, Frequency Domain (emw) ->  lumped port. This will be used for MW excitation. Add one more port for collection.

- In Lumped Port 1, select the circle of the SMA port where input will be applied. Do the same thing for Lumped port 2. For type Use Coaxial.

- Right Click Electromagnetic Waves, Frequency Domain -> Scattering Boundary Condition. Select the 6 faces of the air block.

- Go to Mesh 1 -> Build All

- Go to Study 1 -> Step 1: Frequency Domain. Type the correct Frequency (2.7 GHz here).  Click Compute

- You get the results…

- This figure only shows the electric field distribution. For working with spin defects  we want to see and optimize magnetic field distribution. Right Click Results -> 3D Plot Group.

- In the plot group Right Click -> More Plots -> Multislice

- In the multilice settings change the expression to emw.normB, click Plot.

As an example, here’s a picture of a thin stripline waveguide, optimizing the near surface magnetic field.